Here is the same information in a cleaner interview/reference format:

| Limiting Factor | Description | Profiler Indicators (Nsight Compute) | Typical Remedies |
| --- | --- | --- | --- |
| Memory Bound | The kernel is limited by memory bandwidth. The GPU is moving data as fast as possible, but there is not enough computation per byte of data to keep ALUs fully busy. | • High DRAM bandwidth utilization (close to peak) • Low FLOPS utilization • High memory throughput • ALUs are often waiting for data | • Increase arithmetic intensity (more computation per memory access) • Use tiling/blocking techniques • Improve global memory coalescing • Improve cache utilization (L1/L2) • Fuse kernels to reduce memory traffic |
| Compute Bound | The kernel has enough data available and memory latency is hidden. The bottleneck is the GPU compute units (CUDA cores, Tensor Cores). | • High FLOPS utilization approaching GPU peak • Low memory bandwidth utilization • Compute pipelines are saturated | • Use lower precision (FP16, BF16, FP8, INT8, INT4) • Use Tensor Cores/specialized hardware units • Increase instruction-level parallelism (ILP) • Loop unrolling • Reduce instruction dependencies • Fuse operations • Exploit sparsity |
| Latency Bound | The kernel does not have enough independent work to hide memory or instruction latency. Warps frequently stall waiting for data or dependencies. | • Low achieved memory bandwidth (far below peak) • High "Stall: Wait" / "Stall: Scoreboard" percentage • High "Not Selected" warp stalls • Low SM utilization despite available resources | • Increase occupancy (more active warps per SM) • Increase ILP (multiple independent accumulators) • Loop unrolling • Software prefetching • Pipeline memory loads and computation • Reduce dependency chains |
| Underutilizing the GPU | The GPU resources are not being fully used. There is not enough parallel work, or kernels are too small/short to keep SMs busy. | • Low occupancy • Low FLOPS • Low memory bandwidth utilization • Nsight Systems timeline shows gaps between kernels • Sparse GPU activity | • Increase problem size • Increase batch size • Launch more threads/blocks • Fuse small kernels • Use CUDA streams for overlapping work • Use persistent kernels for repeated workloads |

## Quick Mental Model

| Problem | GPU is waiting for | Main Fix |
| --- | --- | --- |
| Memory Bound | Data movement | Reduce memory traffic / increase reuse |
| Compute Bound | More compute capacity | Optimize math operations |
| Latency Bound | Individual operations finishing | Add parallelism and hide latency |
| Underutilized GPU | More work | Give GPU more tasks |

## Example Mapping to CUDA Kernels

| Kernel Example | Likely Bottleneck | Optimization |
| --- | --- | --- |
| Naive matrix multiplication | Memory bound | Tiling + shared memory |
| Poor reduction kernel | Latency bound | Warp shuffle + more parallel reduction |
| Tensor Core GEMM | Compute bound | Increase Tensor Core utilization |
| Vector addition with small arrays | Underutilized GPU | Increase workload size |
| Attention softmax | Memory bound/latency bound | Fusion + better memory reuse |

For CUDA profiling interviews, this table is one of the most important frameworks because almost every optimization decision starts with: